Introduction:

The PAULMON debugger is my attempt to make a user-friendly 8051

debugger, with enough on-line information that it should be

unnecessary to read this doc file.  PAULMOM is targeted for

use by the microprocessor design course at Oregon State, but

may be used by anyone (who can figure it out) for projects

ranging from research to commercial products.  PAULMON is

free and may not be distributed for profit whatsoever.

Since I don't expect Prof's or TA's at OSU to make students

aware of this documentation nor to provide it nor do I

expect students to read much of it, I wrote PAULMON to

be very simple and to provide lots of on-line clues about

what it can do and how to go about it.  I hope that you

find PAULMON to be useful and easy to use.  Good Luck.

-Paul Stoffregen

(paul@ece.orst.edu)

DISCLAIMER: This is free software.  As far as warranty is

concerned, you get exactly what you pay for!  I have

tried to make this code as good as possible during the

four weeks I worked on it, but nobody is perfect and

portions (the single step in particular) were never well

tested.  USE AT YOUR OWN RISK.  The assembly source is

provided in case there's something you don't like.

ADDITIONAL DISCLAIMER: This doc file has lots of tyopes

and other errorss, and I really don't care.  PAULMON

was written to be easy enough that this file ought to

be unnecessary, but people ask for it nonetheless,

usually before they even try to use the thing.

What You Will Need to use it:

PAULMON is 8051 assembly code which is intended to be

burned into a 2764 EPROM, though a pair of 2732's

could be used or a bigger rom can be used with the

rest being empty or filled with other code.  The

EPROM with PAULMON should be addressed so that it

is read from 0000 to 1FFF with the 8051's EA pin

wired to make it read all code from external memory.

PAULMON uses the built in UART in the 8051 to

communicate with the user.  Typically, a PC computer

is used with a terminal program, an 8051 assembler,

and a text editor to form a simple, low cost 8051

development system with PAULMON.  A serial line

receiver and driver should be used (the MAX232 is

a good choice, IMHO) to interface the 8051 to the

PC's serial port.  Only TxD, RxD and ground are

used (no handshaking) and PAULMON adapts to use

whatever baud rate the computer is using (if it

can with the crystal you select, see below)

PAULMON is intended to be used with RAM as well, and

the default location for the beginning of the RAM is

2000 (hex), right after the EPROM, though the RAM

can be used anywhere in the range of 2000 - FFFF.

The read enable signal to the RAM should be the

logical OR of the RD and PSEN signals, so that read

attempts to external code memory or program memory

spaces will read from the RAM.  (use an AND gate to

do the logical OR of these signals, since they

are active low!)  Obviously the write enable of the

RAM should be connected to the WR pin of the 8051.

Having a RAM connected in this way will allow the

download command in PAULMON to write your program

into the RAM (writing into the external data memory

space).  Then you can run your program, since

read attempts from the external program memory space

will read from the RAM chip.

How to get is set up:

Design and build your 8051 board.  All that is really

required is the 8051, an EPROM, a latch (74xx373),

some sort of address decoding to enable the EPROM

for memory access between 0000-1FFF, and a line

receiver to convert the high voltage RS232 to a

TTL (or CMOS) compatible signal (or else you'll

toast the 8051 before it even has a chance).

To really use PAULMON, a RAM is required as well

as the AND gate to allow both program and data

read cycles to read the RAM memory, and a reset

button to easily get back to PAULMON when your

program crashes.

With just the minimal setup, set the computer's

baud rate to something slow (like 1200 bps) and

power up the board.  Press Enter (Return) and

hopefully you'll see a screenful of text from

PAULMON.  PAULMON does not send line feed

characters, so the terminal emulator software

must be configured to translate CR -> CR/LF.

(PAULMON ignores LF characters is receives.)

If the entire message ends up on one line, then

the terminal is not translating CR -> CR/LF.

After it works, you can try increasing the baud

rate and COLD-BOOTING (you must turn the power

off, taking the reset line high will not make

PAULMON look for the new baud rate... or change

the bytes where it stores the old baud rate...

see the code if you're interested)  If the

minimal system shows no signs of life, it's

time to check the wiring, usually starting by

making sure you didn't swap the TxD and RxD

lines.

The Automatic Baud Rate detection:

This code was borrowed from MDP/51 by Kei-Yong Khoo.  It is run

immediatly after a system reset.  It waits for a <RETURN>

character, and uses it to calculate the timer #1 reload value.

Some modifications have been made to Khoo's code.  It requires

only one character.  It also stores the reload value in four

memory locations in internal ram (78H, 79H, 7AH, and 7BH).  These

four locations are unlikely to be changed during a user program's

execution or while the debugger is running.  When another reset

occurs (without removing the power) the program looks at those

four locations.  If all four agree, then it uses that reload

value and does not require another keypress.  It is interesting

to note that occasionally, with cystal values which produce exact

reload values (such an 7.3728 MHz), the baud rate detection

routine may not correctly calculate the reload value.  Garbage

will get printed all over the screen.  If this happens, just

switch off the power and try again.  The advantage of crystals

such as the 7.3728 Mhz is that they allow transmission at speeds

of 9600 and 19200 baud!  It is highly reccomended that you use

the highest possible baud rate with this debugger, as is tends to

print quite a bit of text to the screen.

On-line Help:

By typing '?' at the main menu, a help screen summarizing the

available commands is printed.  On-line help is also available

regarding the single step run feature.  This help is accessed by

typing '?' just after using the 'R' command.  While in the single

step mode, a summary of commands is also available, again by

typing '?'.

The <ESC> key:

The <ESC> key is supported extensively.  It will abort all

commands from any prompt.  It will stop the list and hex dump

commands in the middle of their printing.  It will also interrupt

the printing of text to the screen!  This is useful at slow baud

rates, since a full screen of text can take quite a while to

print at 300 baud.

The Download Program command (type 'D')

This allows you to send the object code from the assembler to the

external ram.  The object file must be a standard Intel Hex

Format file, such as the .OBJ file created by the Psuedo-

Assembler, by Psuedo-Corp.  The file must be sent as an ASCII

transfer.  The protocal such as XMODEM is used.  Pressing the

<ESC> key at any time will abort the transfer.  Please note that

most communications programs use the <ESC> key to abort their

transfer.  In this is the case, the first <ESC> will halt the

terminal, pressing it again will abort the recieve at the

8051/31. Unlike some other debuggers, PAULMON will recognize

the <ESC> key anywhere in the middle of the incoming data,

not just at the beginning of a line.

The Run Program command (type 'R')

The run command allows you to execute your program.  Two types of

run are supported, Normal and Single-Step.  The single step mode

is explained later, as it is fairly complex.  During a normal

run, the quiviliant of an LCALL to your code is given.  During

the execution of your program, the debugger obviously has no

control of the system, unless of course your program calls one of

the subroutines offered by the degugger in the jump table at

location 0030H.  After specifying which run mode you need, the

location of your program is prompted, with the current memory

pointer value as the default choice.  As is the case at all

prompts, pressing the <ESC> key will abort the run command.  It

is interesting to note that the run command leaves timer #1 in

auto-baud rate generation mode.  If serial communication is

desired at the same baud rate as that used for the debugger,

timer #1 need not be given a new reload value.  It is recommended

that the character input and output routines from the debugger be

used via the jump table.

The New Memory Location command (type 'N')

The debugger operates with a pointer to the data memory with

which you are working.  This pointer is used by the list and hex

dump command.  It is also the default run location.  The pointer

is incremented as memory is viewed or modified.  Just type 'N' to

change it.

The List command (type 'L')

This debugger gives you the ability to list your program's code

directly from the computer's memory.  All the 8051/31 mnemunonics

are supports, as well as the names of the special function

registers.  Bit addressable locations are displayed using the

standard syntax (e.g. PWS.2 or 20.5), but individual bit location

names are not supported (e.g. SCON.0 will print in place of RI).

Obviously, the original labels used in the source code cannot be

printed, instead the memory locations are displayed.  Other

special Intel assembly formats, such as $ and CALL are not

supported.  However, the list command can provide a reassuring

look at the program directly from the memory.

The Hex Dump command (type 'H')

By typing 'H', the next 256 bytes of ram are dumped to the screen

in hex and ascii.  The <ESC> key may be pressed to abort the

printout.

The Edit command (type 'E')

This command allows you to change the values of memory locations

in the external ram.  Each location's old value is shown.  If

<ESC> is pressed, the current location's value is not changed.

The Jump Table:

Desipte the use of the word "jump", the user must LCALL to these

locations!  The individual locations contain jumps to the

subroutines, which all terminate with a RET.  The table provides

the user with a memory location to call to that WILL NOT CHANGE

if the debugger is reassembled.  The routines available are:

0030:   Cout    -Sends the byte in Acc to the

serial port.

0032:   Cin     -Waits for a character from

the serial port, returned in Acc.

0034:   pHex    -Prints the two digit hex value

in Acc to the serial port.

0036:   pHex16  -Prints the four digit hex value

in DPTR to the serial port.

0038:   pString -Prints the string in code memory

pointed to by DPTR to the serial

port.  The string must terminate

with 00H or a high bit set

003A:   gHex    -Gets a two digit hex value from

the serial port, returned in Acc

003C:   gHex16  -Gets a four digit hex value from

the serial port, returned in DPTR

003E:   Esc     -Checks to see if the <ESC> key is

waiting in SBUF.  Clears the buffer

if it is, and returns with the

carry set.  Otherwise, leaves SBUF

untouched, and returns with C=0.

0040:   Upper   -Converts character in Acc to

uppercase if it is lowercase.

0042:   Init    -Automatic baud rate detection.

The memory location can be placed directly in your code, or an

EQU can be used to make your code more readable.  For example:

Program:.EQU    gHex16, 003AH           ;this make the code nice

MOV     DPTR, #StrLoc           ;load DPTR

LCALL   gHex16                  ;print the DPTR

MOV     A,#13

LCALL   0030H                   ;print a <RET>

LCALL   0038H                   ;print the string

RET

StrLoc: .DB     "This is my String.", 0

Most of these routines leave the registers unchanged, however, it

is a good idea to consult the source code just to be sure...

In particular, the phex routine DESTROYS the contents of Acc, so

beware.  (this has cause some people some grief, who had assumed

the phex would leave Acc unchanged.  If you want it unchanged,

the original .ASM file is provided for you to modify)

The Single-Step Run:

[This part was never written, and the single step run code is

somewhat buggy, primarily due to a lack of available beta

testers... so docs were never written, but PAULMON ought to give

you enough clues to figure it out if you try.]