EXAMPLE1.MAP

Simply provides a channel split point at note 60 or C3.

Channel  1  below and including the split point  and  channel  two

above.   There  is  no controller mapping or  patch  change  data.

Notice that no velocity split points are defined.

EXAMPLE2.MAP

This map uses the same key split point.  Notice in the first

key zone that there are three velocity zones defined.  Striking

a note below and including C3 softly will trigger channel 1,

with medium velocity triggers channel 2, and smacking it good

causes  the  note to sound on channel  3.    Notice  the  velocity

adjustments.  This arrangement shrinks the velocity range of each

channel to 1/3 the normal midi range but will provide a medium

output  velocity on all channels in the key  range.   Notice  also

that  the  output  of  velocity zones 1 and  3  is  ranged  by  12

semitones or 1 octave.   Key zone two is identical to that in  the

first map.

There is no patch change data in this map.

Controller  1,  the  modulation wheel,  has  been  mapped  to

controller 7,  volume,  for only midi channel 2.   Notice that  is

will act as the mod wheel for all other channels triggered.

EXAMPLE3.MAP

This map contains 3 key zones.  The first key zone has four

velocity  zones.   The  second key zone has  two  velocity  zones.

Notice that both trigger the same midi channel but that the first

has dropped the pitch by 1 octave.  The third key zone has only

1 velocity zone.

The controller data is the same as EXAMPLE2.MAP with the

exception that pitch bend data is only active on channel 3.

There is patch change data for this map.  Notice that the

map addresses midi channels 1 thru 4 in the zone definitions.

Each channel is assigned a patch change value at performance

startup time (patch 0).  Notice that channel 4 never changes from

its initial setting.  Some synths will not handle the entire range

of patch change data (0 - 127).   The FB-01,  for  instance,  will

only accept values 0 - 47.  Check your midi implementation

charts  if  you get funny results.   After patch  4,  Gozone  will

continue cycling thru the table, although nothing will happen

until it wraps all the way back around to patch 0.